# Oak Rooting-Depth Sensitivity — ESA GDE Analysis

Working technical notes — Vina Subbasin (2027 BC Proposition network)

Prepared for: Tovey Giezentanner   |   Date: 2026-07-17   |   Branch: claude/oak-rooting-depth-gde-ifgslp

Data source: js/gde-centroids-data.js — 1228 GDE polygon centroids, Vina GSA GDE Technical Memo (ESA, March 2026). All figures below computed live from that file.

# 1. Summary

The question: does the repo's ESA GDE information let us change the assumed oak rooting depth (reported at ~18.2 ft) and re-run the 'likely GDE' count for deeper roots (30/40/50 ft), and if so, how many more polygons qualify and where?

Bottom line: yes, for the Spring 90th-percentile scenario — the scenario that produces the 464 count. The repo stores, per polygon, both the rooting depth (adjst) and the 90th-percentile depth-to-groundwater (p90_spring), and the 'likely GDE' flag is exactly root-depth ≥ depth-to-groundwater. Re-thresholding is therefore a defensible re-computation, not new modeling. Changing oak alone from 18.2 ft to 50 ft raises the basin-wide likely count from 464 to 511 (+47).

Three points of correction to the framing as originally stated:

Oak depth confirmed. Oak's rooting depth is 18.17 ft in the data (≈ the 18.2 ft cited) — confirmed.

464 is basin-wide, not oak. 464 is the all-species Spring-90th likely count, not oak-specific. Oak contributes 104 of the 464.

Oak is deepest tree, not deepest overall. Oak is the deepest true tree genus, but two land-cover classes are assigned deeper values: 'river' (20.0 ft) and Artemisia/sagebrush (24.52 ft). Oak is not the single deepest value in the study.

# 2. Verification of the stated claims

| Claim as stated | Finding | Verdict |
| --- | --- | --- |
| Oak rooting depth ≈ 18.2 ft | Quercus adjst = 18.17 ft (constant, all 262 oak polys) | Confirmed |
| ~464 polygons under the 90th pct | Spring-90th likely count = 464 (all species) | Confirmed (all-species) |
| Oak is the deepest rooting depth | river = 20.0, Artemisia = 24.52 ft are deeper | Partly — deepest tree only |
| 18.2 ft intersecting the water table | flag = (adjst ≥ p90_spring); reproduces the flag for all 1228 polys | Confirmed (mechanism) |

Reproduction fidelity: the rule root-depth ≥ depth-to-groundwater matches the published Spring-90th flag for 1228 of 1228 polygons (exact), and the Fall-90th flag for 1088 of 1228.

# 3. How the "likely GDE" flag is computed

Each centroid carries an adjusted rooting depth (adjst, constant per genus) and a 90th-percentile depth-to-groundwater for spring and fall (p90_spring, p90_fall). The 90th percentile is a relatively deep / dry condition of the water-level record, so the test is conservative: a polygon is 'likely GDE' when roots reach down to that deeper water table.

Rule (verified): likely = (adjst ≥ p90). Increasing rooting depth relaxes the test, so polygons over progressively deeper groundwater begin to qualify.

Which scenarios can be re-derived from this repo:

Spring 90th percentile: reproduced exactly by the rule — the 464. Re-thresholding is clean here.

Fall 90th percentile: published flags are STRICTER than the centroid rule (140 polygons where roots would reach water but ESA still flagged not-likely), implying ESA applied additional criteria; not cleanly re-derivable.

2015 / 2021 drought years: cannot be re-derived — the year-specific water depths are not stored in this file (only the spring and fall 90th-percentile depths are).

# 4. Flexibility to adjust rooting depth

High, for the Spring-90th scenario, because both inputs to the rule are in the repo. Two caveats worth stating in any memo that uses these numbers:

It is an assumption, not raw data. adjst is genus-specific and constant; the repo does NOT contain the script that derived 18.17 ft for oak (that lives in the ESA TM and the sibling vina-stream-connectivity repo). Changing it is a modeling-assumption change.

Gains saturate. 111 of the 262 oak polygons sit over groundwater deeper than 50 ft (up to 371 ft). These are foothill / terrace oaks that never intersect the water table at any realistic rooting depth, so the gains saturate.

# 5. Results — how many more polygons qualify (Spring-90th)

Scenario A — change oak only (all other species held at their ESA depths). This is the direct answer to the question.

| Oak rooting depth | Total likely GDE | Δ vs 464 | Oak likely (of 262) | Oak likely area |
| --- | --- | --- | --- | --- |
| 18.2 ft (current) | 464 | — | 104 | 750 ac |
| 30.0 ft | 491 | +27 | 131 | 896 ac |
| 40.0 ft | 503 | +39 | 143 | 1,009 ac |
| 50.0 ft | 511 | +47 | 151 | 1,049 ac |

Scenario B — apply the deeper depth to every species basin-wide. A larger assumption change; included for context only.

| Rooting depth (all genera) | Total likely GDE | Δ vs 464 |
| --- | --- | --- |
| 18.2 ft | 531 | +67 |
| 30.0 ft | 613 | +149 |
| 40.0 ft | 678 | +214 |
| 50.0 ft | 716 | +252 |

# 6. Where the newly-qualifying oak polygons are

All flips fall in the same Sacramento River / lower-creek corridor as the existing GDEs (overall extent lat 39.53–39.88, lon −121.6 to −122.1), in two clusters: the Chico-area reaches (~39.73–39.75 N) and the southern Durham / Llano Seco stretch (~39.64 N).

18 → 30 ft:  +27 oak polygons, 145 ac added.

| Polygon id | lat | lon | area (ac) | spring DTG (ft) |
| --- | --- | --- | --- | --- |
| 45027 | 39.745 | -121.874 | 24.3 | 19.4 |
| 45033 | 39.750 | -121.861 | 14.7 | 21.8 |
| 45031 | 39.751 | -121.849 | 14.6 | 28.4 |
| 57376 | 39.645 | -121.776 | 14.5 | 28.9 |
| 57378 | 39.643 | -121.756 | 10.2 | 25.4 |

30 → 40 ft:  +12 oak polygons, 114 ac added.

| Polygon id | lat | lon | area (ac) | spring DTG (ft) |
| --- | --- | --- | --- | --- |
| 45002 | 39.736 | -121.827 | 45.6 | 34.4 |
| 148768 | 39.709 | -121.825 | 43.1 | 31.8 |
| 57385 | 39.649 | -121.788 | 6.2 | 39.0 |
| 57394 | 39.654 | -121.789 | 4.4 | 38.9 |
| 45032 | 39.751 | -121.842 | 2.6 | 36.0 |

40 → 50 ft:  +8 oak polygons, 40 ac added.

| Polygon id | lat | lon | area (ac) | spring DTG (ft) |
| --- | --- | --- | --- | --- |
| 57398 | 39.669 | -121.764 | 10.7 | 44.1 |
| 45006 | 39.738 | -121.825 | 8.7 | 44.1 |
| 148782 | 39.739 | -121.819 | 8.1 | 47.4 |
| 57483 | 39.733 | -121.815 | 3.4 | 48.2 |
| 45036 | 39.753 | -121.832 | 3.2 | 42.3 |

# 7. Reference — rooting depth by genus/class

| Genus / class | Polygons | Rooting depth (ft) | Area (ac) |
| --- | --- | --- | --- |
| Artemisia | 11 | 24.52 | 48 |
| river | 164 | 20.00 | 424 |
| Quercus | 262 | 18.17 | 1,873 |
| Platanus | 62 | 16.29 | 437 |
| Acer | 84 | 15.97 | 257 |
| Juglans | 47 | 13.19 | 348 |
| Populus | 154 | 11.91 | 1,265 |
| Baccharis | 1 | 11.12 | 10 |
| Rubus | 165 | 9.88 | 522 |
| Arundo | 5 | 9.11 | 8 |
| Salix | 163 | 7.20 | 682 |
| Tamarix | 1 | 7.20 | 2 |
| Flooded Veg | 22 | 7.07 | 108 |
| Alnus | 14 | 5.00 | 74 |
| Typha | 34 | 3.55 | 143 |
| Sambucus | 1 | 3.00 | 2 |
| Leymus | 1 | 2.97 | 1 |
| Schoenoplectus | 5 | 2.20 | 14 |
| Vitus | 20 | 1.00 | 59 |
| Heterotheca | 10 | 1.00 | 83 |
| Persicaria | 2 | 1.00 | 5 |

# 8. Suggested next step

Wire a rooting-depth slider into the existing 'ESA GDE areas' §5.2 overlay (like the LML and MT-raise sliders already in the dashboard): recompute the Spring-90th 'likely' count and re-color the map live as oak rooting depth is dragged from 18 to 50 ft. Scope it to the Spring-90th scenario and label it clearly as an oak-rooting-depth sensitivity — not a re-run of ESA's full methodology (which, for fall and the drought years, adds criteria not reproducible from this repo).

Figures regenerate from js/gde-centroids-data.js; recompute if that file changes. Read-only analysis — no dashboard or data files were modified in producing this note.